Introduction to Microwave Remote Sensing

Microwave remote sensing is a quite effective technique that uses the electromagnetic waves within the range of microwave spectrum (wavelengths ranging from 1mm to 1m) to gather information about the Earth’s surface. This kind of remote sensing is very exceptional because it can penetrate clouds, haze, and even vegetation, allowing it to provide accurate data regardless of weather conditions or time of day. These capabilities make microwave remote sensing valuable for applications where optical remote sensing techniques are less useful, such as for cloudy weather or nighttime observation.

Key Concepts of Microwave Remote Sensing:

- Types of Microwave Remote Sensing:

- Active Microwave Remote Sensing: In active systems, the sensor generates and transmits its own microwave energy toward the Earth's surface and measures the energy reflected back.

Sensor Used:  Synthetic Aperture Radar (SAR)

Applications:

- Analyzing urban growth patterns and monitor land use changes.

- Identifying and tracking oil spills on the ocean surface using the smooth texture of oil against rough water.

- Passive Microwave Remote Sensing: Passive systems do not emit any energy. Instead, they detect natural microwave radiation emitted by objects.

Sensor Used: Radiometer

Applications:

- Meteorology

- Hydrology

- Oceanography

- Microwave Bands and Their Applications:

Microwave remote sensing uses specific frequency bands within microwave spectrum, each with unique properties.

| Bands | Wavelength (in cm) | Properties |
| --- | --- | --- |
| X-Band | 2.4 – 3.75 | High frequency, short wavelength; suitable for detailed surface imaging. |
| C-Band | 3.75 – 7.5 | Moderate penetration capability; balances between detail and surface depth. |
| L-Band | 15 – 30 | Long wavelength; penetrates vegetation and soil, useful for subsurface studies. |
| S-Band | 7.5 – 15 | Medium wavelength; better penetration of atmosphere for weather observations. |
| Ku, Ka Band | 1.1 – 2.4 | Very short wavelength; high resolution for fine-scale surface detection. |

- Microwave Remote Sensing Satellites:

- Radarsat Series: With global environmental monitoring and protection being a worldwide concern, RADARSAT-1 was developed as Canada's flagship to pursue this priority. RADARSAT-1 became the first commercial Earth observation satellite built by Canada for just observing the natural resources of Earth and the changes occurring due to human intervention. It incorporated an advanced synthetic aperture radar instrument that could be steered to collect data over an extremely wide 1175 km swath using seven different beam modes. The applications of RADARSAT-2 range from sea ice mapping and navigation of ships through iceberg detection, agricultural crop monitoring, maritime surveillance for detecting ships and pollution, terrestrial defense surveillance with recognition of the targets, geological mapping, monitoring of mining, and land use analysis, mapping of wetlands, topographic mapping, and the rest.

- Sentinel 1: Sentinel-1 is a project under the Copernicus Earth observation program of the European Space Agency. With its C-band Synthetic Aperture Radar, Sentinel-1 acquires high-resolution images regardless of weather, cloud cover, or even illumination by the sun. The critical applications of Sentinel-1 include land and sea monitoring, flood monitoring, vessel detection, and environmental management. All-weather, day-night imaging capability makes it highly crucial for disaster response as well as sustainable resource management.

- Alos-2: Alos-2 equipped with the PALSAR-2 (Phased Array L-Band Synthetic Aperture Radar) is a Japanese Earth observation satellite developed by JAXA. It operates using the L-band spectrum allowing it to penetrate vegetation and soils hence it is very useful for multiple purposes such as forest monitoring disaster management land deformation studies and agricultural lands mapping. ALOS 2 provides higher resolution plus expanded observation coverage compared to ALOS-1 thus giving more crucial data in providing services on global environmental change observation and resource management.

- Applications of Microwave Remote Sensing:

1. Flood Mapping: It detects and monitors flooded areas, providing near-real-time information for emergency management.

2. Soil Moisture Monitoring: Measures soil moisture levels critical for agriculture and drought assessment.

3. Sea Ice Monitoring-Tracks the extent, thickness, and drift of sea ice are monitored for navigation as well as climate research.

4. Urban Deformation Monitoring: It detects land subsidence and deformation of infrastructure, such as building or bridge stability.